# TECHNICAL ORDER 1042

VLAN ASSIGNMENT - MARCEE1
These IDs are yours. Build this database on both switches: a VLAN database is locally significant, so AN-2 will not learn AN-1's VLANs across the trunk

| VLAN | NAME | PURPOSE |
| --- | --- | --- |
| 310 | Management | IT workstation + server |
| 120 | Reception | Reception PC + printer |
| 330 | Data | Second-floor offices |
| 180 | Native | Trunk native VLAN – mitigates VLAN hopping |
| 888 | Dead_Vlan | Suspended. Unused ports quarantined here. State: suspend. |

Trunk (Fa0/1 both switches): 802.1Q, allowed 310,120,330, native 180. Every unused port: access VLAN 888 + shutdown.