# Layer 2 Instructor Required Command Checklist

Student commands and instructor-checkable outcomes from Part 2 pages 9 through 15

# Purpose

This is the narrowed Layer 2 scope for the TSS2 capstone. It lists what the student is required to perform on AN-1 and AN-2 and what the instructor is required to verify. VLAN IDs and most VLAN names are seeded in the student's Binder.

The required flow is:

Harden both switches.

Create the five VLANs on each switch.

Assign AN-1 access ports, trunks, and unused ports.

Assign AN-2 access ports, trunk, and unused ports.

Verify both switches and save both configurations.

# Device hardening on both switches

enable
configure terminal
hostname AN-1
enable secret <Binder enable secret>
service password-encryption
no ip domain-lookup
banner motd #WARNING: DoD System#

line console 0
 password <Binder console password>
 login
 exit

line vty 0 4
 password <Binder VTY password>
 login
 exit

Use hostname AN-2 when configuring the second switch.

| Instructor check | Required result |
| --- | --- |
| Identity | Hostname is AN-1 or AN-2 as appropriate. |
| Privileged access | Enable secret is configured; legacy enable password is not used. |
| Console and VTY | Each line has the Binder password and login enabled. |
| Security services | Password encryption is enabled and DNS lookup is disabled. |
| Banner | The DoD warning banner is present with the required text. |

# VLAN database on both switches

vlan <MGMT ID>
 name <Management name>
 exit
vlan <RECEP ID>
 name <Reception name>
 exit
vlan <DATA ID>
 name <Data name>
 exit
vlan <NATIVE ID>
 name <Native name>
 exit
vlan <DEAD ID>
 name Dead_Vlan
 state suspend
 exit

| Instructor check | Required result |
| --- | --- |
| Database completeness | All five VLANs exist on both AN-1 and AN-2. |
| Seeded values | IDs and role names match the student's Binder exactly. |
| Unused VLAN | The VLAN is named Dead_Vlan and is suspended. |
| Independence | AN-2 has its own VLAN database; the trunk does not replicate VLAN configuration. |

# AN-1 switchports

## Access ports

interface fa0/2
 switchport mode access
 switchport access vlan <MGMT ID>
 exit
interface range fa0/3 - 4
 switchport mode access
 switchport access vlan <RECEP ID>
 exit
interface fa0/5
 switchport mode access
 switchport access vlan <MGMT ID>
 exit

| Ports | Required assignment |
| --- | --- |
| Fa0/2 | Management |
| Fa0/3 - Fa0/4 | Reception |
| Fa0/5 | Management |

## Trunks

interface fa0/1
 switchport mode trunk
 switchport trunk allowed vlan <MGMT>,<RECEP>,<DATA>
 switchport trunk native vlan <NATIVE ID>
 exit
interface fa0/6
 switchport mode trunk
 switchport trunk allowed vlan <MGMT>,<RECEP>,<DATA>
 switchport trunk native vlan <NATIVE ID>
 exit

Fa0/1 carries VLANs between AN-1 and AN-2.

Fa0/6 carries VLANs from AN-1 toward CN-1.

The native VLAN is not included in the allowed VLAN list.

## Unused ports

interface range fa0/7 - 8
 switchport mode access
 switchport access vlan <DEAD ID>
 shutdown
 exit

# AN-2 switchports

## Access ports

interface fa0/2
 switchport mode access
 switchport access vlan <DATA ID>
 exit
interface fa0/3
 switchport mode access
 switchport access vlan <DATA ID>
 exit
interface fa0/4
 switchport mode access
 switchport access vlan <DATA ID>
 exit

| Ports | Required assignment |
| --- | --- |
| Fa0/2 | Data |
| Fa0/3 | Data |
| Fa0/4 | Data |

## Trunk and unused ports

interface fa0/1
 switchport mode trunk
 switchport trunk allowed vlan <MGMT>,<RECEP>,<DATA>
 switchport trunk native vlan <NATIVE ID>
 exit
interface range fa0/5 - 8
 switchport mode access
 switchport access vlan <DEAD ID>
 shutdown
 exit

AN-2 Fa0/1 must match the AN-1 trunk allowed list and native VLAN.

AN-2 Fa0/5 through Fa0/8 must be assigned to Dead_Vlan and shut down.

# Layer 2 verification and save

show vlan brief
show interfaces trunk
copy running-config startup-config

| Switch | Required actions | Instructor checks |
| --- | --- | --- |
| AN-1 | Run both show commands and save. | Five VLANs, correct port membership, Dead_Vlan ports down, both trunks correct, configuration saved. |
| AN-2 | Run both show commands and save. | Five VLANs, correct port membership, Dead_Vlan ports down, matching trunk, configuration saved. |

The instructor should require the student to read the output against the Binder, not merely execute the show commands.

# Not part of the Part 2 Layer 2 check

These commands may exist in the broader simulator, but they are not required for the Layer 2 task described in the guide:

ip address
no ip address
ip route
no ip route
encapsulation dot1q
switchport trunk encapsulation dot1q
show ip route
show ip interface brief
show interfaces
show mac address-table
ping
enable password

# Current implementation watch points

The required command syntax is mostly present in the current processor.

show vlan brief currently reports VLANs as active even when Dead_Vlan was suspended.

show vlan brief does not clearly expose that Dead_Vlan ports are shut down.

show interfaces trunk does not fully distinguish configured trunks from operationally active trunks.

The current CLI accepts extra Layer 3 commands on switches, but those commands should not expand the Layer 2 evaluation scope.

## Source

Net Capstone Instructor Guide (2), Part 2 - Layer 2, pages 9-15. Reviewed October 20 2026.