# Naming Standard

Technical Order 1042

# Rack Naming and Mounting Standard

AN = Access Node (switch). CN = Core Node (router). Rack order is not cosmetic: power at the top, away from data; patch panel directly above the switch it feeds; router below the switch it uplinks to; and the heavy box lowest for center of gravity.

# First Floor Rack

| Slot Range | Device | Name | Why |
| --- | --- | --- | --- |
| Rear U38 – U35 | Rack PDU | PDU1 | Power at the top rear, away from data cabling |
| Front U40 | 24-port Patch Panel | Patch1 | Directly above its switch – short patch runs |
| Front U38 – U35 | 24-port Switch | AN-1 | Access node, ground floor |
| Front U34 – U38 | Core Router | CN-1 | Below the switch it trunks to; feeds the demarc |
| Front U8 – U3 | Rack Server | IT2 | Heavy and hot – lowest in the rack |

# Second Floor Rack

| Slot | Device | Name |
| --- | --- | --- |
| Rear U38 – U35 | Rack PDU | PDU2 |
| Front U40 | Patch Panel | Patch2 |
| Front U38 – U35 | 24-port Switch | AN-2 |

No server upstairs. Wall mounts are already installed — facilities did that.